Documentation for xspips.h — XSpiPs Driver API Overview

Overview

This file contains the implementation interface for the XSpiPs SPI driver for the Zynq Processing System SPI peripheral. It supports both master and slave modes of operation, providing comprehensive SPI communication features.

The driver handles data transmission/reception including interrupt-driven and polled transfers and supports detailed configuration of SPI parameters.

Key Concepts and Features

- Full-duplex SPI bus: Data is sent and received simultaneously.

- Master-slave communication: Master controls the clock and slave responds.

- Flexible Chip Select and Start modes: Manual or automatic control over CS and transmission start.

- Interrupt-based and polled data transfers: Allows responsiveness and efficiency.

- Error detection: Mode faults, FIFO underflow/overflow, and overrun conditions handled.

- RTOS and processor independent: Designed for use across different environments without dependency.

Configuration Options (Bit Masks)

These bit masks control SPI operation modes and features:

- XSPIPS_MASTER_OPTION (0x01)
Sets SPI device as master. Default is slave.

- XSPIPS_CLK_ACTIVE_LOW_OPTION (0x02)
Configures clock polarity to active low (clock idles high).

- XSPIPS_CLK_PHASE_1_OPTION (0x04)
Clock phase option to sample data on the second edge of the clock.

- XSPIPS_DECODE_SSELECT_OPTION (0x08)
Enables slave select decoding for up to 16 slaves.

- XSPIPS_FORCE_SSELECT_OPTION (0x10)
Manually forces Slave Select signal active.

- XSPIPS_MANUAL_START_OPTION (0x20)
Manual start mode where software controls transmission start.

SPI Clock Prescaler Options

Set the master SPI baud rate as a fraction of the peripheral clock (pclk):

- XSPIPS_CLK_PRESCALE_4 to XSPIPS_CLK_PRESCALE_256
Divide clock by 4, 8, 16, 32, 64, 128, or 256 respectively.

Callback Events

Events passed to callback functions to notify application of SPI driver status:

- XSPIPS_EVENT_MODE_FAULT (1) — Mode fault error occurred.

- XSPIPS_EVENT_TRANSFER_DONE (2) — Data transfer completed successfully.

- XSPIPS_EVENT_TRANSMIT_UNDERRUN (3) — Transmit FIFO underflow (empty).

- XSPIPS_EVENT_RECEIVE_OVERRUN (4) — Receive FIFO overrun (data loss).

Key Data Types

- XSpiPs_StatusHandler:
Callback prototype invoked on SPI events. Takes a callback reference, status event code, and byte count transferred.

- XSpiPs_Config:
Holds static configuration data such as device ID, base address, and input clock frequency.

- XSpiPs:
SPI driver instance containing configuration, transfer buffers, state flags (busy, ready), slave select info, and callback handler references.

Common Macros to Check Driver Options

- XSpiPs_IsManualStart(InstancePtr)
Returns TRUE if manual start is enabled.

- XSpiPs_IsManualChipSelect(InstancePtr)
Returns TRUE if manual chip select is enabled.

- XSpiPs_IsDecodeSSelect(InstancePtr)
Returns TRUE if slave select decoding is enabled.

- XSpiPs_IsMaster(InstancePtr)
Returns TRUE if SPI is operating in master mode.

Watermark and Slave Idle Register Access

- XSpiPs_SetSlaveIdle(InstancePtr, Value) / XSpiPs_GetSlaveIdle(InstancePtr)
Set/get number of idle clock cycles a slave waits before next transfer.

- XSpiPs_SetTXWatermark(InstancePtr, Value) / XSpiPs_GetTXWatermark(InstancePtr)
Set/get watermark level for TX FIFO (range 1–128 bytes).

- XSpiPs_SetRXWatermark(InstancePtr, Value) / XSpiPs_GetRXWatermark(InstancePtr)
Set/get watermark level for RX FIFO (range 1–128 bytes).

Device Enable/Disable

- XSpiPs_Enable(InstancePtr)
Enables the SPI device, allowing transactions.

- XSpiPs_Disable(InstancePtr)
Disables the SPI device.

Function Prototypes

- XSpiPs_LookupConfig(DeviceId)
Retrieves the configuration data for a specified device instance.

- XSpiPs_CfgInitialize(InstancePtr, ConfigPtr, EffectiveAddr)
Initializes the SPI driver instance with configuration data and effective register base address.

- XSpiPs_Reset(InstancePtr)
Resets the SPI hardware and internal state.

- XSpiPs_Transfer(InstancePtr, SendBufPtr, RecvBufPtr, ByteCount)
Starts an interrupt-driven SPI transfer (full-duplex) of ByteCount bytes.

- XSpiPs_PolledTransfer(InstancePtr, SendBufPtr, RecvBufPtr, ByteCount)
Performs a blocking polled SPI transfer.

- XSpiPs_SetStatusHandler(InstancePtr, CallBackRef, FunctionPtr)
Sets the callback function to handle asynchronous driver events.

- XSpiPs_InterruptHandler(InstancePtr)
Interrupt service routine to handle SPI interrupts.

- XSpiPs_Abort(InstancePtr)
Aborts an ongoing SPI transfer.

- XSpiPs_SetSlaveSelect(InstancePtr, SlaveSel) / XSpiPs_GetSlaveSelect(InstancePtr)
Set/get the active slave select line.

- XSpiPs_SelfTest(InstancePtr)
Runs a self-test routine to verify SPI operation.

- XSpiPs_SetOptions(InstancePtr, Options) / XSpiPs_GetOptions(InstancePtr)
Set/get configuration options (e.g., master mode, clock polarity, manual start).

- XSpiPs_SetClkPrescaler(InstancePtr, Prescaler) / XSpiPs_GetClkPrescaler(InstancePtr)
Set/get clock prescaler for SPI speed configuration.

- XSpiPs_SetDelays(InstancePtr, DelayNss, DelayBtwn, DelayAfter, DelayInit) / XSpiPs_GetDelays(InstancePtr, ...)
Configure/get various delay parameters for SPI timing.