Documentation for xspips_hw.h

Overview

This header file contains register offsets, bit masks, constants, macros, and function prototypes used in the low-level SPI driver responsible for interfacing with the SPI hardware in the Zynq Processing System.

The file primarily defines hardware register mappings and provides basic macros for read/write operations to these SPI registers.

Register Offsets

Each SPI device register is accessible via an offset from the SPI base address.

- XSPIPS_CR_OFFSET (0x00) — Configuration Register to control SPI device settings.

- XSPIPS_SR_OFFSET (0x04) — Interrupt Status Register that holds flags indicating interrupt conditions.

- XSPIPS_IER_OFFSET (0x08) — Interrupt Enable Register to enable specific SPI interrupts.

- XSPIPS_IDR_OFFSET (0x0C) — Interrupt Disable Register to disable specific SPI interrupts.

- XSPIPS_IMR_OFFSET (0x10) — Interrupt Mask Register that shows which interrupts are enabled.

- XSPIPS_ER_OFFSET (0x14) — Enable/Disable Register controlling SPI device activation.

- XSPIPS_DR_OFFSET (0x18) — Delay Register to program timing delays, especially helpful in slave mode.

- XSPIPS_TXD_OFFSET (0x1C) — Transmit Data Register where data to be sent over SPI is written.

- XSPIPS_RXD_OFFSET (0x20) — Receive Data Register from which data received over SPI can be read.

- XSPIPS_SICR_OFFSET (0x24) — Slave Idle Count Register specifying idle count for slave mode.

- XSPIPS_TXWR_OFFSET (0x28) — Transmit FIFO Watermark register defining watermark level for TX FIFO.

- XSPIPS_RXWR_OFFSET (0x2C) — Receive FIFO Watermark register defining watermark level for RX FIFO.

Configuration Register (XSPIPS_CR_OFFSET) Bit Masks

This register configures the SPI controller features:

- Mode Fault Generation Enable (XSPIPS_CR_MODF_GEN_EN_MASK): Enables detection of mode faults.

- Manual Transmission Start (XSPIPS_CR_MANSTRT_MASK): Manually triggers SPI transaction start.

- Manual Transmission Start Enable (XSPIPS_CR_MANSTRTEN_MASK): Enables manual start mode.

- Force Slave Select (XSPIPS_CR_SSFORCE_MASK): Forces the slave select signal active.

- Slave Select Decode (XSPIPS_CR_SSCTRL_MASK): Controls which slave select lines are active.

- Slave Select Decode Enable (XSPIPS_CR_SSDECEN_MASK): Enables hardware decoding of slave select signals.

- Prescaler (XSPIPS_CR_PRESC_MASK): Sets clock prescaler for SPI clock speed.

- Clock Phase (XSPIPS_CR_CPHA_MASK) & Polarity (XSPIPS_CR_CPOL_MASK): Configure clock timing.

- Master Enable (XSPIPS_CR_MSTREN_MASK): Enables master mode operation.

SPI Interrupt Masks

The SPI module supports several interrupt types indicated by bits in status and control registers:

- Transmit FIFO Underflow (XSPIPS_IXR_TXUF_MASK): Indicates TX FIFO underrun.

- Receive FIFO Full (XSPIPS_IXR_RXFULL_MASK): RX FIFO has reached full condition.

- Receive FIFO Not Empty (XSPIPS_IXR_RXNEMPTY_MASK): RX FIFO contains data.

- Transmit FIFO Full (XSPIPS_IXR_TXFULL_MASK): TX FIFO is full.

- Transmit FIFO Overwater (XSPIPS_IXR_TXOW_MASK): TX FIFO watermark overflowed.

- Mode Fault (XSPIPS_IXR_MODF_MASK): Mode fault detected.

- Receive FIFO Overrun (XSPIPS_IXR_RXOVR_MASK): Data lost due to RX FIFO overrun.

Enable Register (XSPIPS_ER_OFFSET)

- Controls whether the SPI device is enabled or disabled.

- The bit XSPIPS_ER_ENABLE_MASK set to 1 enables the SPI device.

Delay Register (XSPIPS_DR_OFFSET) Timing Delays

Uses bit fields to program timing delays:

- NSS Delay (XSPIPS_DR_NSS_MASK) — Delay in clock cycles for slave select de-assertion between words.

- Between Transfers Delay (XSPIPS_DR_BTWN_MASK) — Delay between SPI transfers.

- After Transfers Delay (XSPIPS_DR_AFTER_MASK) — Delay after SPI transfers.

- Initial Delay (XSPIPS_DR_INIT_MASK) — Delay before starting SPI transactions.

Slave Idle Count Register (XSPIPS_SICR_OFFSET)

- Specifies the count of peripheral clock cycles the slave waits in CPHA=1 mode before detecting a new SPI transaction start.

FIFO Watermark Registers (XSPIPS_TXWR_OFFSET, XSPIPS_RXWR_OFFSET)

- Define FIFO thresholds for generating interrupts or DMA requests.

- FIFO depth is 128 bytes.

- Reset values are 1 for both TX and RX watermarks.

Macros for Register Access

- XSpiPs_ReadReg(BaseAddress, RegOffset):
Reads a 32-bit SPI register at the specified offset from the base address.

- XSpiPs_WriteReg(BaseAddress, RegOffset, RegisterValue):
Writes a 32-bit value to the SPI register at the specified offset.

Function Prototype

- void XSpiPs_ResetHw(u32 BaseAddress);

  - Resets the SPI hardware module at the given base address.